PAPER – II          PRACTICAL NO. : 04

AIM :  1. IMPLEMENT GRE TUNNEL

2. IMPLEMENT VTP

3. IMPLEMENT NAT

ROLL NO. : 02                                BATCH : M.SC PART-I

DATE : 05/11/22

GRE:

Router 1 :

Router#show ip int br

Interface              IP-Address      OK? Method Status                Protocol

FastEthernet0/0        192.168.1.1     YES manual up                    up

FastEthernet0/1        unassigned      YES unset  administratively down down

Serial0/0/0            10.0.0.1        YES manual up                    up

Serial0/0/1            unassigned      YES unset  administratively down down

Vlan1                  unassigned      YES unset  administratively down down

Router#en

Router#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#int tunnel 1

%LINK-5-CHANGED: Interface Tunnel1, changed state to up

Router(config-if)#ip address 50.50.50.1 255.255.255.0

Router(config-if)#tunnel source f0/0

Router(config-if)#exit

Router(config)#int tunnel 1

Router(config-if)#tunnel destination 20.0.0.2

%LINEPROTO-5-UPDOWN: Line protocol on Interface Tunnel1, changed state to up

Router(config-if)#end

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#show ip int br

Interface              IP-Address      OK? Method Status                Protocol

FastEthernet0/0        192.168.1.1     YES manual up                    up

FastEthernet0/1        unassigned      YES unset  administratively down down

Serial0/0/0            10.0.0.1        YES manual up                    up

Serial0/0/1            unassigned      YES unset  administratively down down

Tunnel1                50.50.50.1      YES manual up                    up

Vlan1                  unassigned      YES unset  administratively down down

Router#

Router 2 :

Router#show ip int br

Interface              IP-Address      OK? Method Status                Protocol

FastEthernet0/0        171.15.10.1     YES manual up                    up

FastEthernet0/1        unassigned      YES unset  administratively down down

Serial0/0/0            unassigned      YES unset  administratively down down

Serial0/0/1            20.0.0.2        YES manual up                    up

Vlan1                  unassigned      YES unset  administratively down down

Router#en

Router#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#int tunnel 1

%LINK-5-CHANGED: Interface Tunnel1, changed state to up

Router(config-if)#tunnel source f0/0

Router(config-if)#exit

Router(config)#int tunnel 1

Router(config-if)#tunnel destination 10.0.0.1

%LINEPROTO-5-UPDOWN: Line protocol on Interface Tunnel1, changed state to up

Router(config-if)#end

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#show ip int br

Interface              IP-Address      OK? Method Status                Protocol

FastEthernet0/0        171.15.10.1     YES manual up                    up

FastEthernet0/1        unassigned      YES unset  administratively down down

Serial0/0/0            unassigned      YES unset  administratively down down

Serial0/0/1            20.0.0.2        YES manual up                    up

Tunnel1                50.50.50.2      YES manual up                    up

Vlan1                  unassigned      YES unset  administratively down down

Router#

VTP:

Server :

Switch>en

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#switchport mode trunk

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

Switch(config-if)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

VTP Version                     : 2

Configuration Revision          : 0

Maximum VLANs supported locally : 255

Number of existing VLANs        : 5

VTP Operating Mode              : Server

VTP Domain Name                 :

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0x7D 0x5A 0xA6 0x0E 0x9A 0x72 0xA0 0x3A

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Local updater ID is 0.0.0.0 (no valid interface found)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#vtp mode server

Device mode already VTP SERVER.

Switch(config)#vtp domain VTPServer

Changing VTP domain name from NULL to VTPServer

Switch(config)#vlan 10

Switch(config-vlan)#Name GJC

Switch(config-vlan)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vlan

Transparent Server :

Switch>en

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#switchport mode trunk

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

Switch(config-if)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

VTP Version                     : 2

Configuration Revision          : 0

Maximum VLANs supported locally : 255

Number of existing VLANs        : 5

VTP Operating Mode              : Server

VTP Domain Name                 :

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0x7D 0x5A 0xA6 0x0E 0x9A 0x72 0xA0 0x3A

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Local updater ID is 0.0.0.0 (no valid interface found)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#vtp mode transparent

Device mode already VTP TRANSPARENT.

Switch(config)#vtp domain VTPTransparent

Changing VTP domain name from NULL to VTPTransparent

Switch(config)#vlan 10

Switch(config-vlan)#Name University

Switch(config-vlan)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vlan

Client :

Switch>en

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#switchport mode trunk

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

Switch(config-if)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

VTP Version                     : 2

Configuration Revision          : 0

Maximum VLANs supported locally : 255

Number of existing VLANs        : 5

VTP Operating Mode              : Server

VTP Domain Name                 : VTPClient

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0xC0 0x46 0xBB 0x26 0x20 0x8B 0xB1 0x44

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Local updater ID is 0.0.0.0 (no valid interface found)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#vtp mode client

Setting device to VTP CLIENT mode.

Switch(config)#vtp domain VTPClient

Domain name already set to VTPClient.

Switch(config)#end

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

S1 :

Switch#en

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#switchport mode trunk

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

Switch(config-if)#exit

Switch(config)#int f0/2

Switch(config-if)#switchport mode trunk

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up

Switch(config-if)#exit

Switch(config)#end

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

VTP Version                     : 2

Configuration Revision          : 0

Maximum VLANs supported locally : 255

Number of existing VLANs        : 5

VTP Operating Mode              : Server

VTP Domain Name                 :

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0x7D 0x5A 0xA6 0x0E 0x9A 0x72 0xA0 0x3A

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Local updater ID is 0.0.0.0 (no valid interface found)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#vtp mode server

Device mode already VTP SERVER.

Switch(config)#vtp domain VTPServer

Changing VTP domain name from NULL to VTPServer

Switch(config)#end

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

VTP Version                     : 2

Configuration Revision          : 0

Maximum VLANs supported locally : 255

Number of existing VLANs        : 5

VTP Operating Mode              : Server

VTP Domain Name                 : VTPServer

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0xB8 0x02 0x0F 0x8B 0xD8 0xFA 0x0B 0x13

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Local updater ID is 0.0.0.0 (no valid interface found)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#vlan 10

Switch(config-vlan)#Name GJC

Switch(config-vlan)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

S2 :

Switch>en

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#end

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

VTP Version                     : 2

Configuration Revision          : 2

Maximum VLANs supported locally : 255

Number of existing VLANs        : 6

VTP Operating Mode              : Server

VTP Domain Name                 : VTPServer

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0xE0 0x55 0x40 0xC9 0x16 0xD4 0xEC 0xBC

Configuration last modified by 0.0.0.0 at 3-1-93 00:14:18

Local updater ID is 0.0.0.0 (no valid interface found)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#vtp mode client

Setting device to VTP CLIENT mode.

Switch(config)#vtp domain VTPClient

Changing VTP domain name from VTPServer to VTPClient

Switch(config)#exit

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

VTP Version                     : 2

Configuration Revision          : 0

Maximum VLANs supported locally : 255

Number of existing VLANs        : 6

VTP Operating Mode              : Client

VTP Domain Name                 : VTPClient

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0x1B 0x59 0xC6 0x3B 0xFF 0xEF 0x8A 0x18

Configuration last modified by 0.0.0.0 at 3-1-93 00:14:18

Switch#show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/2, Fa0/3, Fa0/4, Fa0/5

Fa0/6, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/11, Fa0/12, Fa0/13

Fa0/14, Fa0/15, Fa0/16, Fa0/17

Fa0/18, Fa0/19, Fa0/20, Fa0/21

Fa0/22, Fa0/23, Fa0/24, Gig1/1

Gig1/2

10   GJC                              active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

10   enet  100010     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

S3 :

Switch>en

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#exit

Switch(config)#end

Switch#

%SYS-5-CONFIG_I: Configured from console by console

show vtp status

VTP Version                     : 2

Configuration Revision          : 2

Maximum VLANs supported locally : 255

Number of existing VLANs        : 6

VTP Operating Mode              : Server

VTP Domain Name                 : VTPServer

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0xE0 0x55 0x40 0xC9 0x16 0xD4 0xEC 0xBC

Configuration last modified by 0.0.0.0 at 3-1-93 00:14:18

Local updater ID is 0.0.0.0 (no valid interface found)

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#int f0/1

Switch(config-if)#vtp mode transparent

Setting device to VTP TRANSPARENT mode.

Switch(config)#vtp domain VTPTransparent

Changing VTP domain name from VTPServer to VTPTransparent

Switch(config)#end

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vtp status

VTP Version                     : 2

Configuration Revision          : 0

Maximum VLANs supported locally : 255

Number of existing VLANs        : 6

VTP Operating Mode              : Transparent

VTP Domain Name                 : VTPTransparent

VTP Pruning Mode                : Disabled

VTP V2 Mode                     : Disabled

VTP Traps Generation            : Disabled

MD5 digest                      : 0x96 0xCA 0x57 0x93 0x70 0xF4 0x83 0x36

Configuration last modified by 0.0.0.0 at 3-1-93 00:14:18

Switch#show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/2, Fa0/3, Fa0/4, Fa0/5

Fa0/6, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/11, Fa0/12, Fa0/13

Fa0/14, Fa0/15, Fa0/16, Fa0/17

Fa0/18, Fa0/19, Fa0/20, Fa0/21

Fa0/22, Fa0/23, Fa0/24, Gig1/1

Gig1/2

10   GJC                              active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

10   enet  100010     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0